% Figure 2: Baseline Correction Performance Comparison

figure('Position', [100, 100, 800, 600]);

% Sample wavenumbers

wavenumbers = 200:2:1800;

% True baseline (for reference)

true_baseline = 0.2 + 0.003 * wavenumbers - 1e-6 * wavenumbers.^2;

% Add characteristic Raman peaks

peak_positions = [500, 800, 1050, 1320, 1580];

peak_intensities = [1.2, 0.8, 1.8, 1.4, 2.2];

peak_widths = [30, 25, 35, 30, 40];

% Construct clean Raman spectrum

clean_spectrum = zeros(size(wavenumbers));

for i = 1:length(peak_positions)

clean_spectrum = clean_spectrum + peak_intensities(i) * ...

exp(-((wavenumbers - peak_positions(i)) / peak_widths(i)).^2);

end

% Create raw spectrum with baseline

raw_spectrum = clean_spectrum + true_baseline;

% Polynomial baseline estimation using selected non-peak regions

poly_fit_idx = [1:100, 650:750, length(wavenumbers)-99:length(wavenumbers)];

poly_coeffs = polyfit(wavenumbers(poly_fit_idx), raw_spectrum(poly_fit_idx), 3);

poly_baseline = polyval(poly_coeffs, wavenumbers);

poly_corrected = raw_spectrum - poly_baseline;

% AIRPLS baseline approximation (simulated)

airpls_baseline = true_baseline + 0.15 * sin(wavenumbers / 200);

airpls_corrected = raw_spectrum - airpls_baseline;

% DNN (CNN-LSTM) corrected result (simulated with noise)

dnn_corrected = clean_spectrum + 0.01 * randn(size(clean_spectrum)); % small noise

% Plotting

hold on;

plot(wavenumbers, raw_spectrum, 'k-', 'LineWidth', 1.5);                          % Raw

plot(wavenumbers, poly_corrected + 4, 'b-', 'LineWidth', 1.5);                    % Polynomial

plot(wavenumbers, airpls_corrected + 8, 'g-', 'LineWidth', 1.5);                  % AIRPLS

plot(wavenumbers, dnn_corrected + 12, 'r-', 'LineWidth', 1.5);                    % CNN-LSTM

plot(wavenumbers, clean_spectrum + 16, 'm-', 'LineWidth', 1.5);                   % Ground Truth

% Add baselines (offset dashed lines for visual reference)

plot(wavenumbers, true_baseline, 'k--', 'LineWidth', 1);                          % True baseline

plot(wavenumbers, zeros(size(wavenumbers)) + 4, 'b--', 'LineWidth', 1);          % Poly ref

plot(wavenumbers, zeros(size(wavenumbers)) + 8, 'g--', 'LineWidth', 1);          % AIRPLS ref

plot(wavenumbers, zeros(size(wavenumbers)) + 12, 'r--', 'LineWidth', 1);         % DNN ref

plot(wavenumbers, zeros(size(wavenumbers)) + 16, 'm--', 'LineWidth', 1);         % GT ref

% Customize plot

xlabel('Raman Shift (cm^{-1})', 'FontSize', 12, 'FontWeight', 'bold');

ylabel('Intensity (a.u.)', 'FontSize', 12, 'FontWeight', 'bold');

title('Baseline Correction Performance Comparison', 'FontSize', 14, 'FontWeight', 'bold');

legend({'Raw Spectrum', 'Polynomial Correction', 'AIRPLS Correction', ...

'CNN-LSTM Correction', 'Ground Truth'}, 'Location', 'NorthWest');

grid on;

set(gca, 'FontSize', 11);

xlim([min(wavenumbers), max(wavenumbers)]);

ylim([-1, 19]);

yticks([0, 4, 8, 12, 16]);

yticklabels({'Raw', 'Polynomial', 'AIRPLS', 'CNN-LSTM', 'Ground Truth'});

% Figure 3: SNR Improvement Analysis

figure('Position', [100, 100, 800, 600]);

% Define SNR levels and sample categories

snr_levels = {'Very Low (SNR<10)', 'Low (10≤SNR<30)', 'Medium (30≤SNR<60)', 'High (SNR≥60)'};

categories = {'Pharmaceutical', 'Biological', 'Materials'};

% SNR improvement percentages by method and SNR level

traditional_methods = [42.3, 58.7, 69.5, 74.8;  % Pharmaceutical

28.6, 45.2, 62.1, 70.6;  % Biological

51.8, 65.3, 73.4, 78.2]; % Materials

cnn_lstm_method = [84.6, 91.3, 94.7, 96.2;      % Pharmaceutical

76.5, 85.9, 91.4, 94.5;      % Biological

88.7, 93.2, 95.8, 97.1];     % Materials

% -------- Subplot 1: SNR Improvement by Initial SNR Level --------

subplot(2, 1, 1);

x = categorical(snr_levels);

y_trad = mean(traditional_methods, 1); % mean across categories

y_cnn = mean(cnn_lstm_method, 1);

err_trad = std(traditional_methods, 0, 1);

err_cnn = std(cnn_lstm_method, 0, 1);

b = bar(x, [y_trad' y_cnn'], 'grouped');

b(1).FaceColor = [0.4, 0.4, 0.4];

b(2).FaceColor = [0.8, 0.2, 0.2];

% Add error bars manually

hold on;

ngroups = size(y_trad, 2);

nbars = size(b, 2);

groupwidth = min(0.8, nbars/(nbars + 1.5));

for i = 1:length(x)

x_group = double(i);

% Traditional

errorbar(x_group - groupwidth/2 + groupwidth/(2*nbars), y_trad(i), err_trad(i), 'k.', 'LineWidth', 1.5);

% CNN-LSTM

errorbar(x_group + groupwidth/2 - groupwidth/(2*nbars), y_cnn(i), err_cnn(i), 'k.', 'LineWidth', 1.5);

end

% Customize plot

title('Average SNR Improvement by Initial SNR Level', 'FontSize', 14, 'FontWeight', 'bold');

ylabel('SNR Improvement (%)', 'FontSize', 12, 'FontWeight', 'bold');

ylim([0, 100]);

grid on;

legend('Traditional Methods', 'CNN-LSTM (Ours)', 'Location', 'SouthEast');

set(gca, 'FontSize', 11);

% -------- Subplot 2: SNR Improvement by Sample Category --------

subplot(2, 1, 2);

x = categorical(categories);

y_trad_cat = mean(traditional_methods, 2); % mean across SNR levels

y_cnn_cat = mean(cnn_lstm_method, 2);

err_trad_cat = std(traditional_methods, 0, 2);

err_cnn_cat = std(cnn_lstm_method, 0, 2);

b = bar(x, [y_trad_cat y_cnn_cat], 'grouped');

b(1).FaceColor = [0.4, 0.4, 0.4];

b(2).FaceColor = [0.8, 0.2, 0.2];

% Add error bars manually

hold on;

for i = 1:length(x)

x_group = double(i);

% Traditional

errorbar(x_group - groupwidth/2 + groupwidth/(2*nbars), y_trad_cat(i), err_trad_cat(i), 'k.', 'LineWidth', 1.5);

% CNN-LSTM

errorbar(x_group + groupwidth/2 - groupwidth/(2*nbars), y_cnn_cat(i), err_cnn_cat(i), 'k.', 'LineWidth', 1.5);

end

% Customize plot

title('Average SNR Improvement by Sample Category', 'FontSize', 14, 'FontWeight', 'bold');

ylabel('SNR Improvement (%)', 'FontSize', 12, 'FontWeight', 'bold');

ylim([0, 100]);

grid on;

legend('Traditional Methods', 'CNN-LSTM (Ours)', 'Location', 'SouthEast');

set(gca, 'FontSize', 11);

% Figure 4: Peak Detection Accuracy Analysis

figure('Position', [100, 100, 800, 500]);

%% ------------------- Spectrum Preprocessing Comparison -------------------

% Define wavenumber range

wavenumbers = 500:2:1800;

% Generate synthetic clean spectrum

clean_spectrum = zeros(size(wavenumbers));

peak_positions = [650, 750, 950, 1050, 1300, 1580];  % True peak positions

peak_intensities = [0.8, 0.6, 1.2, 0.7, 1.4, 1.6];

peak_widths = [30, 25, 40, 25, 35, 30];

for i = 1:length(peak_positions)

clean_spectrum = clean_spectrum + ...

peak_intensities(i) * exp(-((wavenumbers - peak_positions(i)) / peak_widths(i)).^2);

end

% Add baseline and noise

baseline = 0.5 + 0.0008 * wavenumbers - 1e-7 * wavenumbers.^2;

noise_level = 0.15;

noisy_spectrum = clean_spectrum + baseline + noise_level * randn(size(wavenumbers));

% Traditional preprocessing (with imperfections)

traditional_baseline = 0.45 + 0.001 * wavenumbers - 1.2e-7 * wavenumbers.^2;

traditional_result = noisy_spectrum - traditional_baseline;

traditional_result = traditional_result + 0.08 * randn(size(traditional_result)) - 0.03;

% CNN-LSTM result (close to clean spectrum)

cnn_lstm_result = clean_spectrum + 0.03 * randn(size(clean_spectrum));

% Plot spectra

subplot(2, 1, 1);

plot(wavenumbers, noisy_spectrum, 'k-', 'LineWidth', 1);

hold on;

plot(wavenumbers, traditional_result + 2, 'b-', 'LineWidth', 1);

plot(wavenumbers, cnn_lstm_result + 4, 'r-', 'LineWidth', 1);

plot(wavenumbers, clean_spectrum + 6, 'g-', 'LineWidth', 1);

% Annotate true peaks

for i = 1:length(peak_positions)

xline(peak_positions(i), 'k--', 'LineWidth', 0.8);

end

% Customize plot

title('Spectrum Preprocessing Comparison', 'FontSize', 14, 'FontWeight', 'bold');

xlabel('Raman Shift (cm^{-1})', 'FontSize', 12, 'FontWeight', 'bold');

ylabel('Intensity (a.u.)', 'FontSize', 12, 'FontWeight', 'bold');

legend({'Raw Spectrum', 'Traditional Processing', 'CNN-LSTM Processing', 'Ground Truth'}, ...

'Location', 'NorthWest');

grid on;

set(gca, 'FontSize', 11);

ylim([-0.5, 8]);

yticks([0, 2, 4, 6]);

yticklabels({'Raw', 'Traditional', 'CNN-LSTM', 'Ground Truth'});

%% ------------------- Peak Detection Accuracy -------------------

subplot(2, 1, 2);

% Peak intensity vs. accuracy

peak_intensities_norm = [0.1, 0.2, 0.4, 0.6, 0.8, 1.0];

traditional_accuracy = [31.2, 58.7, 76.4, 85.9, 91.7, 94.2];

cnn_lstm_accuracy = [68.5, 83.7, 92.8, 96.3, 98.5, 99.4];

% Standard deviations

traditional_std = [12.5, 10.2, 8.6, 7.1, 5.8, 4.5];

cnn_lstm_std = [9.8, 7.3, 5.5, 3.8, 2.3, 1.1];

% Error region fill (Traditional)

x_fill = [peak_intensities_norm, fliplr(peak_intensities_norm)];

y_fill_trad = [traditional_accuracy + traditional_std, ...

fliplr(traditional_accuracy - traditional_std)];

fill(x_fill, y_fill_trad, 'b', 'FaceAlpha', 0.2, 'EdgeColor', 'none');

hold on;

% Error region fill (CNN-LSTM)

y_fill_cnn = [cnn_lstm_accuracy + cnn_lstm_std, ...

fliplr(cnn_lstm_accuracy - cnn_lstm_std)];

fill(x_fill, y_fill_cnn, 'r', 'FaceAlpha', 0.2, 'EdgeColor', 'none');

% Accuracy line plots

plot(peak_intensities_norm, traditional_accuracy, 'bo-', ...

'LineWidth', 2, 'MarkerSize', 8, 'MarkerFaceColor', 'b');

plot(peak_intensities_norm, cnn_lstm_accuracy, 'ro-', ...

'LineWidth', 2, 'MarkerSize', 8, 'MarkerFaceColor', 'r');

% Customize plot

xlabel('Normalized Peak Intensity', 'FontSize', 12, 'FontWeight', 'bold');

ylabel('Peak Detection Accuracy (%)', 'FontSize', 12, 'FontWeight', 'bold');

title('Peak Detection Accuracy vs. Peak Intensity', 'FontSize', 14, 'FontWeight', 'bold');

legend('Traditional Methods', 'CNN-LSTM (Ours)', 'Location', 'SouthEast');

grid on;

set(gca, 'FontSize', 11);

ylim([0, 100]);

*********************************************************************************

% Figure 5: Downstream Classification Analysis

figure('Position', [100, 100, 800, 700]);

%% ---------------- Classification Accuracy by Sample Category ----------------

categories = {'Pharmaceutical', 'Biological', 'Materials', 'Overall'};

methods = {'Raw', 'Polynomial', 'AIRPLS', 'Wavelet', 'CNN-LSTM'};

accuracies = [65.3, 79.8, 83.5, 81.7, 95.6;    % Pharmaceutical

57.8, 74.2, 78.1, 76.5, 93.2;    % Biological

72.1, 85.6, 88.5, 87.3, 97.4;    % Materials

65.1, 79.9, 83.4, 81.8, 95.4];   % Overall average

subplot(2, 2, 1);

b1 = bar(categorical(categories), accuracies, 'grouped');

colormap(jet(length(methods)));

% Add value labels

for i = 1:size(accuracies, 1)

for j = 1:size(accuracies, 2)

if accuracies(i,j) > 70

text(b1(j).XData(i), accuracies(i,j) + 1, sprintf('%.1f', accuracies(i,j)), ...

'HorizontalAlignment', 'center', 'FontSize', 8);

end

end

end

title('Classification Accuracy by Sample Category', 'FontSize', 14, 'FontWeight', 'bold');

ylabel('Accuracy (%)', 'FontSize', 12, 'FontWeight', 'bold');

ylim([50, 100]);

grid on;

legend(methods, 'Location', 'SouthEast', 'FontSize', 9);

set(gca, 'FontSize', 11);

%% ---------------- F1 Score by Sample Category ----------------

f1_scores = [61.2, 77.5, 81.3, 79.6, 94.8;    % Pharmaceutical

52.7, 71.6, 75.9, 74.3, 92.5;    % Biological

70.3, 84.2, 86.9, 85.7, 96.8;    % Materials

61.4, 77.8, 81.4, 79.9, 94.7];   % Overall average

subplot(2, 2, 2);

b2 = bar(categorical(categories), f1_scores, 'grouped');

colormap(jet(length(methods)));

% Add value labels

for i = 1:size(f1_scores, 1)

for j = 1:size(f1_scores, 2)

if f1_scores(i,j) > 70

text(b2(j).XData(i), f1_scores(i,j) + 1, sprintf('%.1f', f1_scores(i,j)), ...

'HorizontalAlignment', 'center', 'FontSize', 8);

end

end

end

title('F1 Score by Sample Category', 'FontSize', 14, 'FontWeight', 'bold');

ylabel('F1 Score (%)', 'FontSize', 12, 'FontWeight', 'bold');

ylim([50, 100]);

grid on;

legend(methods, 'Location', 'SouthEast', 'FontSize', 9);

set(gca, 'FontSize', 11);

%% ---------------- Confusion Matrices ----------------

classes = {'Class A', 'Class B', 'Class C', 'Class D'};

traditional_cm = [82, 9, 7, 2; 11, 79, 6, 4; 8, 7, 81, 4; 5, 8, 10, 77];  % AIRPLS

cnn_lstm_cm = [95, 3, 1, 1; 2, 96, 1, 1; 2, 1, 94, 3; 1, 2, 3, 94];       % CNN-LSTM

% Normalize to percentages

traditional_cm_norm = traditional_cm ./ sum(traditional_cm, 2) * 100;

cnn_lstm_cm_norm = cnn_lstm_cm ./ sum(cnn_lstm_cm, 2) * 100;

% Custom subplot positions for better matrix layout

pos1 = [0.13, 0.1, 0.35, 0.35];

pos2 = [0.57, 0.1, 0.35, 0.35];

% --- Traditional Method Confusion Matrix

axes('Position', pos1);

imagesc(traditional_cm_norm);

colormap('jet');

caxis([0, 100]);

colorbar;

title('Traditional Method (AIRPLS)', 'FontSize', 12, 'FontWeight', 'bold');

xlabel('Predicted Class', 'FontSize', 10);

ylabel('True Class', 'FontSize', 10);

set(gca, 'XTick', 1:length(classes), 'XTickLabel', classes, 'FontSize', 9);

set(gca, 'YTick', 1:length(classes), 'YTickLabel', classes, 'FontSize', 9);

% Add text values

for i = 1:size(traditional_cm_norm, 1)

for j = 1:size(traditional_cm_norm, 2)

text(j, i, sprintf('%.1f', traditional_cm_norm(i,j)), ...

'HorizontalAlignment', 'center', 'FontSize', 9, 'Color', 'w');

end

end

% --- CNN-LSTM Confusion Matrix

axes('Position', pos2);

imagesc(cnn_lstm_cm_norm);

colormap('jet');

caxis([0, 100]);

colorbar;

title('CNN-LSTM Method (Ours)', 'FontSize', 12, 'FontWeight', 'bold');

xlabel('Predicted Class', 'FontSize', 10);

ylabel('True Class', 'FontSize', 10);

set(gca, 'XTick', 1:length(classes), 'XTickLabel', classes, 'FontSize', 9);

set(gca, 'YTick', 1:length(classes), 'YTickLabel', classes, 'FontSize', 9);

% Add text values

for i = 1:size(cnn_lstm_cm_norm, 1)

for j = 1:size(cnn_lstm_cm_norm, 2)

text(j, i, sprintf('%.1f', cnn_lstm_cm_norm(i,j)), ...

'HorizontalAlignment', 'center', 'FontSize', 9, 'Color', 'w');

end

end